1. RIP (Routing Information Protocol)

enable

configure terminal

router rip

version 2

no auto-summary             ! Critical for VLSM support

network [Classful_Network]  ! e.g., network 192.168.10.0

exit

2. EIGRP (Enhanced Interior Gateway Routing Protocol)

router eigrp [AS_Number]     ! AS Number must match neighbors

no auto-summary

! Advertise networks

network [Network_ID] [Wildcard_Mask]

! Example: network 192.168.10.0 0.0.0.255

exit

3. OSPF (Open Shortest Path First)

router ospf [Process_ID]

! Network command formats:

! network [IP_Address] [Wildcard] area [Area_ID]

! Example (Place interface in Area 0):

network 192.168.10.0 0.0.0.255 area 0

! Example (Default Route Injection):

default-information originate always

exit

4. BGP (Border Gateway Protocol)

router bgp [Your_AS_Number]

bgp router-id [x.x.x.x]	! just for debugging

! Define Neighbor (Pointer to the OTHER router's IP)

neighbor [Neighbor_gateway_IP] remote-as [Neighbor_AS]

! Advertise Your Networks (Must exist in routing table exactly)

network [Your_Network_IP] mask [Subnet_Mask]

exit

5. NAT (Network Address Translation)

- Assign IPs to PCs and router interfaces.

- Apply these on the router translating NAT (Near to the PCs):

interface FastEthernet0/0      ! Your LAN / Server connection

ip nat inside

exit

interface Serial0/0/0          ! Your Internet / WAN connection

ip nat outside

exit

- Do this for every private IP and give different public IP respectively from the public network

en

config t

ip nat inside source static [private_ip] [public_ip]

! Example: Map internal Server (192.168.10.5) to Public IP (50.50.50.5)

ip nat inside source static 192.168.10.5 50.50.50.5

- Do static routing (When using fake address like 200.200.200.0, write the new fake address as destination_network)

ip route [destination_network] [subnet_mask] [next_hop_ip]

6. DHCP (Dynamic Host Configuration Protocol)

Dynamic Allocation (Standard Pool)

- Assign IPs to DHCP server and router interface (NOT to PCs).

- Assign DHCP pools for every network.

- Then, do this for each network having PC:

DHCP Relay (Helper Address)

- Used on Routers that don't have the server, pointing to the server.

interface FastEthernet0/0      ! Interface facing the Client PC

ip helper-address [IP_of_DHCP_Server]

7. VLANs (Virtual LANs)

On Switch:

! Create VLAN

vlan 10

name Sales

exit

! Assign Port to VLAN (Access)

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

! Configure Trunk (Connection to Router/Switch)

interface FastEthernet0/24

switchport mode trunk

8. ACL (Access Control Lists)

ACL_No: 1-99 (Standard ACL)

access-list [ACL_No] deny host [Bad_PC_IP]

access-list [ACL_No] deny [Network address] [Wildcard mask]

access-list [ACL_No] permit any        ! Essential!

! Apply to interface

interface FastEthernet0/0

ip access-group [ACL_No] in/out

(in/out means going in/out to this interface)