# Lab 09

# Topic:

- Irvine setup

- Procedure

- Stack operations

# Procedure/Functions

### Procedure Declaration and Definition in Assembly

In assembly language, a procedure (also called a subroutine or function) is a block of code that performs a specific task. Procedures are useful for modularizing code and avoiding repetition. The Irvine32 library includes built-in procedures like WriteString, but you can also define your own.

#### Structure of a Procedure

- Declaration: A procedure is declared by giving it a name followed by the keyword PROC.

- Definition: The actual code of the procedure is written between the declaration (PROC) and the termination (ENDP).

- Return: The procedure typically ends with a RET instruction, which tells the CPU to return to the calling code.

#### Key Elements

- PROC: Marks the beginning of the procedure.

- ENDP: Marks the end of the procedure.

- RET: Returns control to the calling function.

- CALL: The instruction to invoke the procedure.

#### Simple Example

This example demonstrates a procedure called DisplayMessage that prints a message to the console using the Irvine32 library.

### Code Example (main.asm)

INCLUDE Irvine32.inc      			; Include the Irvine32 library

.data

message BYTE "Hello from the procedure!", 0                ; Define a message string

.code

main PROC

call DisplayMessage                                                    ; Call the DisplayMessage procedure

exit                                                                               ; Exit the program

main ENDP

; Define the DisplayMessage procedure

DisplayMessage PROC

mov edx, OFFSET message                                ; Load the address of the message into EDX

call WriteString                                    ; Print the string using Irvine's WriteString procedure

call CrLf               			  ; Move to the next line

ret                      			; Return to the calling function

DisplayMessage ENDP      		     ; End of the procedure

END main                   			 ; End of the program

# Stack Applications

There are several important uses of runtime stacks in programs:

  - A stack makes a convenient temporary save area for registers when they are used for more than one purpose. After they are modified, they can be restored to their original values.

  - When the CALL instruction executes, the CPU saves the current subroutine’s return address on the stack.

  - When calling a subroutine, you pass input values called arguments by pushing them on the stack.

  - The stack provides temporary storage for local variables inside subroutines.

## PUSH Instruction:

The PUSH instruction first decrements ESP and then copies a source operand into the stack.

A 16-bit operand causes ESP to be decremented by 2. A 32-bit operand causes ESP to be decremented by 4. There are three instruction formats:

PUSH reg/mem16

PUSH reg/mem32

PUSH imm32

## POP Instruction:

The POP instruction first copies the contents of the stack element pointed to by ESP into a 16- or

32-bit destination operand and then increments ESP. If the operand is 16 bits, ESP is incremented by 2; if the operand is 32 bits, ESP is incremented by 4:

POP reg/mem16

POP reg/mem32

Tasks:

### 1. Reversing an Array using Stack

- Problem Statement: Write an assembly program to reverse an array of integers using stack operations. Push each element of the array onto the stack and then pop them in reverse order.

- Input: [3, 5, 7, 9, 11]

- Output: [11, 9, 7, 5, 3]

Code:

include irvine32.inc

.DATA

arr db 3, 5, 7, 9, 11

arrs = lengthof arr

.CODE

reverse Proc

mov ecx, arrs

mov esi, offset arr

mov eax, 0

L1:

mov al, byte ptr [esi]

push eax

inc esi

Loop L1

mov ecx, arrs

mov esi, offset arr

L2:

pop [esi]

inc esi

Loop L2

ret

reverse Endp

main Proc

call reverse

mov ecx, arrs

mov edi, 0

L3:

movzx eax, arr[edi]			; Move the address of the string into EDX

call WriteInt			; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

inc edi

Loop L3

exit						; Exit the program

main endp

end main

### 2. Checking Balanced Parentheses

- Problem Statement: Write an assembly program to check if a string of parentheses is balanced using a stack. For each opening parenthesis (, push it onto the stack, and for each closing parenthesis ), pop from the stack and check if it matches.

- Input: "((())())"

- Output: "Balanced"

Code:

include irvine32.inc

.DATA

arr db "((())())", 0

str1 db "Balanced", 0

str2 db "Not balanced", 0

br1 = "("

br2 = ")"

bool db 0

temp dd 0

.CODE

isValid Proc

pop temp

mov bl, br1

mov bh, br2

mov dl, 0

mov eax, 0

mov esi, offset arr

mov [esp], dl

cmp [esi], dl

je finish

mov ecx, lengthof arr

L1:

cmp [esi], bl

je J1

jmp else1

J1:

mov al, byte ptr [esi]

push eax

inc esi

jmp skip

else1:

cmp [esi], bh

je J2

jmp else2

J2:

cmp [esp], dl	;if stack is empty

je Jumps

jmp elses

Jumps:

jmp finish

elses:

pop edi

else2:

inc esi

skip:

Loop L1

cmp [esp], dl	;if stack is empty

je J3

jmp else3

J3:

mov bl, 1

mov bool, bl

else3:

finish:

push temp

ret

isValid Endp

main Proc

call isValid

mov dl, 1

cmp bool, dl

je yes

jmp no

yes:

mov edx, offset str1			; Move the address of the string into EDX

jmp nooo

no:

mov edx, offset str2			; Move the address of the string into EDX

nooo:

call WriteString			; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

exit						; Exit the program

main endp

end main

### 3. Palindrome Checker

- Problem Statement: Write an assembly program to check whether a given string is a palindrome. Use the stack to reverse the string and compare it with the original.

- Input: "madam"

- Output: "Palindrome"

Code:

include irvine32.inc

.DATA

arr db "madam", 0

arrl = lengthof arr - 1

iseven db 0

address dd ?

output1 db "Palindrome", 0

output2 db "Not a palindrome", 0

.CODE

Ispalindrome Proc

pop address

mov eax, 0

mov [esp], eax

mov [esp - 4], eax

; ---Check if length is even or odd---

mov eax, 0

mov edx, 0

mov ax, arrl

mov bx, 2

div bx

mov ebx, eax

cmp edx, 0

je J1

jmp else1

J1:

mov dl, 1

mov iseven, dl

else1:

; Pushing half string

mov ecx, ebx

mov esi, offset arr

mov dx, 0

L1:

mov dl, [esi]

push dx

inc esi

Loop L1

;;;;;

mov dl, 1

cmp iseven, dl

je hehe

inc esi

hehe:

mov ecx, ebx

mov edx, 0

L2:

mov dl, [esi]

cmp dx, [esp]

je J2

jmp else2

J2:

pop dx

else2:

inc esi

Loop L2

mov edx, 0

cmp dx, [esp]

je yes

jmp no

yes:

mov bx, 1

jmp finish

no:

mov bx, 0

finish:

push address

ret

IsPalindrome Endp

main Proc

Call IsPalindrome

mov ax, 0

cmp bx, ax

je nopal

mov edx, offset output1			; Move the address of the string into EDX

call WriteString			; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

jmp boo

nopal:

mov edx, offset output2			; Move the address of the string into EDX

call WriteString			; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

boo:

exit						; Exit the program

main endp

end main

### 4. Evaluating a Postfix Expression

- Problem Statement: Implement a postfix expression evaluator using stack operations. Push operands onto the stack and pop them when encountering an operator to perform calculations.

- Input: "5 6 + 2 *" (Infix notation for (5 + 6) * 2)

- Output: 22

Code:

include irvine32.inc

.DATA

;;arr db "9 2 - 2 / 5 * 7 - 9 - 9 -", 0

arr db "5 6 + 2 *", 0

arrl = lengthof arr - 1

answer dd ?

str1 db "Output: ", 0

address dd ?

.CODE

solvePostfix Proc

pop address

mov eax, 0

mov [esp], eax

mov [esp - 4], eax

mov ecx, arrl

mov ebx, 0

mov esi, offset arr

L1:

mov bl, 20h			;SPACE ' '

cmp [esi], bl

je skip			;if space comes

;Checking for operators

mov bh, "+"

cmp [esi], bh

je operatorhandle

mov bh, "-"

cmp [esi], bh

je operatorhandle

mov bh, "*"

cmp [esi], bh

je operatorhandle

mov bh, "/"

cmp [esi], bh

je operatorhandle

mov bh, "%"

cmp [esi], bh

je operatorhandle

;Push operand

mov ebx, 0

mov bl, [esi]

sub bx, 48

push ebx

jmp skip

operatorhandle:

;    '+'    ;

cmp bh, "+"

jne minus

pop ebx

pop eax

add eax, ebx

push eax

jmp skip

;    '-'    ;

minus:

cmp bh, "-"

jne multiply

pop ebx

pop eax

sub eax, ebx

push eax

jmp skip

;    '*'    ;

multiply:

cmp bh, "*"

jne divide

pop ebx

pop eax

mul ebx

push eax

jmp skip

;    '/'    ;

divide:

cmp bh, "/"

jne modulus

mov edx, 0

pop ebx

pop eax

div ebx

push eax

jmp skip

;    '%'    ;

modulus:

cmp bh, "%"

jne skip

mov edx, 0

pop ebx

pop eax

div ebx

;mov al, ah

;mov ah, 0

push edx

jmp skip

skip:

inc esi

dec ecx

cmp ecx, 0

jne L1

;Loop L1

mov eax, [esp]

mov answer, eax

push address

ret

solvePostfix Endp

main Proc

call solvePostfix

mov edx, offset str1			; Move the address of the string into EDX

call WriteString			; Call the WriteString procedure to print the string

mov eax, answer				; Move the address of the string into EDX

call WriteInt				; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

exit						; Exit the program

main endp

end main

### 5. Sorting an Array using Stack (Bubble Sort)

- Problem Statement: Use stack operations (PUSH and POP) to simulate swapping in a Bubble Sort algorithm to sort an array.

- Input: [8, 4, 3, 7, 6]

- Output: [3, 4, 6, 7, 8]

Code:

include irvine32.inc

.DATA

arr dw 8, 4, 3, 7, 6

address dd ?

.CODE

bubblesort Proc

AgainWeGo:

mov cx, 0

mov [ebp+4], cx			;Reset swap local var to 0

dec dword ptr [ebp+8]	;decreasing size

mov esi, [ebp+12]		;moving address of array

mov ebx, 0				;initialize array index to zero

Loop1:

mov ax,[esi]		;load number to ax

cmp ax,[esi+2]		;compare with next number

jle noswap				;no swap if already inorder

mov dx,[esi+2]		;load second element in dx

mov [esi+2],ax		;store first number in second

mov [esi],dx		;store second number in first

mov dword ptr [ebp+4], 1				;flag that a swap has been done

noswap:

inc bx				;advance bx to next index

add esi, 2				;incrementing pointer

cmp bx, [ebp+8]				;are we at last index

jne loop1				;if not than compare next 2

cmp dword ptr [ebp+4],1				;check if swap has been done

je AgainWeGo				;if yes than make another pass

ret 8

bubblesort Endp

main Proc

mov eax, 0

push offset arr

push lengthof arr

sub esp, 4

push ebp

mov ebp, esp

call bubblesort

add esp, 8

mov ecx, lengthof arr

mov edi, 0

L3:

movzx eax, arr[edi]			; Move the address of the string into EDX

call WriteInt			; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

add edi, 2

Loop L3

exit						; Exit the program

main endp

end main

### 6. Infix to Postfix Conversion

- Problem Statement: Convert an infix expression (e.g., 3 + 4 * 2) to postfix notation (e.g., 3 4 2 * +) using stack operations.

- Input: "3 + 4 * 2"

- Output: "3 4 2 * +"

include irvine32.inc

.DATA

arr db "3 + 4 * 2", 0

arrl = lengthof arr - 1

outarr db arrl dup(?), 0

temp db 30 dup(0)

str1 db "Output: ", 0

address dd ?

.CODE

convert Proc

pop address

mov eax, 0

mov [esp], eax

mov [esp - 4], eax

mov ecx, arrl

mov ebx, 0

mov esi, offset arr

mov edi, offset outarr

mov ebp, -1

L1:

;Checking for operators

mov bh, "+"

cmp [esi], bh

je operatorhandle

mov bh, "-"

cmp [esi], bh

je operatorhandle

mov bh, "*"

cmp [esi], bh

je operatorhandle

mov bh, "/"

cmp [esi], bh

je operatorhandle

mov bh, "%"

cmp [esi], bh

je operatorhandle

;Cancatenate operand

mov bl, [esi]

mov [edi], bl

inc edi

jmp skip

operatorhandle:

inc esi

dec ecx

;    '+'    ;

cmp bh, "+"

jne minus

mov dh, "-"

cmp [esp], dh

je equalprecltr

mov dh, "*"

cmp [esp], dh

je lowerprec

mov dh, "/"

cmp [esp], dh

je lowerprec

mov dh, "%"

cmp [esp], dh

je lowerprec

jmp higherprec

jmp skip

;    '-'    ;

minus:

cmp bh, "-"

jne multiply

mov dh, "+"

cmp [esp], dh

je equalprecltr

mov dh, "*"

cmp [esp], dh

je lowerprec

mov dh, "/"

cmp [esp], dh

je lowerprec

mov dh, "%"

cmp [esp], dh

je lowerprec

jmp higherprec

jmp skip

;    '*'    ;

multiply:

cmp bh, "*"

jne divide

mov dh, "+"

cmp [esp], dh

je higherprec

mov dh, "-"

cmp [esp], dh

je higherprec

mov dh, "/"

cmp [esp], dh

je equalprecltr

mov dh, "%"

cmp [esp], dh

je equalprecltr

jmp higherprec

jmp skip

;    '/'    ;

divide:

cmp bh, "/"

jne modulus

mov dh, "+"

cmp [esp], dh

je higherprec

mov dh, "-"

cmp [esp], dh

je higherprec

mov dh, "*"

cmp [esp], dh

je equalprecltr

mov dh, "%"

cmp [esp], dh

je equalprecltr

jmp higherprec

jmp skip

;    '%'    ;

modulus:

cmp bh, "%"

jne skip

mov dh, "+"

cmp [esp], dh

je higherprec

mov dh, "-"

cmp [esp], dh

je higherprec

mov dh, "/"

cmp [esp], dh

je equalprecltr

mov dh, "*"

cmp [esp], dh

je equalprecltr

jmp higherprec

;;;;;;;;;;;

higherprec:

cmp ebp, -1

jne lowerprec2

mov bl, bh

mov bh, 0

push bx

jmp skip

;;;;;;;;;;;

lowerprec:

pop dx

inc ebp

mov temp[ebp], dl

jmp operatorhandle

jmp skip

lowerprec2:

mov bl, bh

mov bh, 0

push bx

mov bx, 0

mov bl, temp[ebp]

mov [edi], bx

dec ebp

cmp ebp, -1

jne lowerprec2

jmp skip

;;;;;;;;;;;;;

equalprecltr:

pop dx

inc ebp

mov temp[ebp], dl

mov bl, bh

mov bh, 0

push bx

jmp skip

skip:

inc esi

dec ecx

cmp ecx, 0

jne L1

;Loop L1

;;;Cancatenate all stack

canst:

mov dx, [esp]

cmp dx, 0

je theend

mov dx, 20h

mov [edi], dx

inc edi

pop dx

mov [edi], dx

inc edi

jmp canst

theend:

mov eax, [esp]

push address

ret

convert Endp

main Proc

call convert

mov edx, offset str1			; Move the address of the string into EDX

call WriteString			; Call the WriteString procedure to print the string

mov edx, offset outarr				; Move the address of the string into EDX

call WriteString				; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

exit						; Exit the program

main endp

end main

### 7. Implementing a Stack Using Arrays

- Problem Statement: Simulate a stack data structure using an array. Implement the basic stack operations: PUSH, POP, PEEK, and DISPLAY.

- Input: Push: 3, Push: 5, Pop, Push: 9

- Output: [3, 9]

Code:

include irvine32.inc

.DATA

arr dw 101 dup(?)

arrsize dw ?

inputval dw ?

str1 db "Enter the size of stack (0 - 100): ", 0

str2 db "Out of range... Enter again (0 - 100): ", 0

emptymess db "Oh no! The stack is empty :( ", 0

fullmess db "Oh no! The stack is full :( ", 0

disp1 db "Stack: [ ", 0

disp2 db ", ", 0

disp3 db " ]", 0

peek1 db "Peek: ", 0

.CODE

CreateStack Proc

mov eax, offset arr - 2

mov ebp, eax

;Message

mov edx, offset str1      ; Move the address of the string into EDX

call WriteString	      ; Call the WriteString procedure to print the string

jmp TakeInput

AgainMessage:

mov edx, offset str2      ; Move the address of the string into EDX

call WriteString	      ; Call the WriteString procedure to print the string

TakeInput:

mov eax, 0

mov ax, arrsize

call ReadInt

mov arrsize, ax

call CrLf

;----- Input Validation -----

cmp arrsize, bx

jl AgainMessage

cmp arrsize, cx

jg AgainMessage

ret

CreateStack Endp

IsFull Proc

mov dx, 0

mov eax, offset arr

mov ebx, 0

mov bx, arrsize	;Only length

dec bx			;L-1

add bx, bx		;(L-1)*type arr	 (bx*2)

add eax, ebx

cmp ebp, eax

jne notfull

mov dx, 1

notfull:

ret

IsFull Endp

IsEmpty Proc

mov dx, 0

mov eax, offset arr

sub eax, 2

cmp ebp, eax

jne notempty

mov dx, 1

notempty:

ret

IsEmpty Endp

Pushst Proc

Call isFull

cmp dx, 1

je itsfull

add ebp, 2

mov dx, inputval

mov [ebp], dx

jmp ohh

itsfull:

mov edx, offset fullmess

call WriteString

call CrLf

ohh:

ret

Pushst Endp

Popst Proc

Call isEmpty

cmp dx, 1

je itsempty

sub ebp, 2

jmp yaaay

itsempty:

mov edx, offset emptymess

call WriteString

call CrLf

yaaay:

ret

Popst Endp

Peekst Proc

Call isEmpty

cmp dx, 1

je itsempty

mov edx, offset peek1

Call WriteString

mov eax, 0

mov ax, [ebp]

call WriteInt

call CrLf

jmp yaay

itsempty:

mov edx, offset emptymess

call WriteString

call CrLf

yaay:

ret

Peekst Endp

Display Proc

mov edx, 0

mov eax, 0

mov edi, offset arr

mov edx, offset disp1

Call WriteString

Call isEmpty

cmp dx, 1

je itsempty

mov eax, 0

PrintL:

mov ax, [edi]

Call WriteInt

cmp edi, ebp

je Exitploop

mov edx, offset disp2

Call WriteString

add edi, 2

jmp PrintL

Exitploop:

itsempty:

mov edx, offset disp3

Call WriteString

Call CrLf

ret

Display Endp

main Proc

mov bx, 0

mov cx, 100

Call CreateStack

mov bx, 3

mov inputval, bx

Call Pushst

mov bx, 5

mov inputval, bx

Call Pushst

mov bx, 7

mov inputval, bx

Call Pushst

mov bx, 6

mov inputval, bx

Call Pushst

mov bx, 4

mov inputval, bx

Call Pushst

mov bx, 2

mov inputval, bx

Call Pushst

Call Popst

Call Popst

mov bx, 2

mov inputval, bx

Call Pushst

Call Peekst

Call Display

Call Popst

Call Popst

Call Popst

Call Popst

Call Popst

Call Peekst

Call Display

exit						; Exit the program

main endp

end main

### 8. Finding the Maximum Element in a Stack

- Problem Statement: Write a program that finds the maximum element in a stack. After pushing all elements, pop them to check the maximum value.

- Input: [2, 9, 4, 6, 1]

- Output: 9

Code:

include irvine32.inc

.DATA

str1 db "Max = ", 0

arr db 2, 9, 4, 6

arrl = lengthof arr

max db ?

temp dd ?

address dd ?

.CODE

Findmax Proc

pop address

mov esi, offset arr

mov ecx, arrl

L1:

mov eax, 0

mov al, [esi]

push eax

inc esi

Loop L1

mov ah, [esp]

;Largest find

mov ecx, lengthof arr

L2:

pop temp

mov ebx, temp

cmp bl, ah

jg J1

jmp else1

J1:

mov ah, bl

else1:

Loop L2

mov max, ah

push address

ret

Findmax Endp

main Proc

call Findmax

mov edx, offset str1			; Move the address of the string into EDX

call WriteString

movzx eax, max			; Move the address of the string into EDX

call WriteInt			; Call the WriteString to print the string

call CrLf						; Move to the next line after printing

exit						; Exit the program

main endp

end main

### 9. Checking Stack Overflow and Underflow

- Problem Statement: Implement a stack with a fixed size using an array. The program should check for stack overflow (when pushing an element onto a full stack) and stack underflow (when popping from an empty stack).

- Input: Stack size = 5, push 6 elements

- Output: "Stack Overflow"

Code:

include irvine32.inc

.DATA

arr dw 101 dup(?)

arrsize dw ?

inputval dw ?

str1 db "Enter the size of stack (0 - 100): ", 0

str2 db "Out of range... Enter again (0 - 100): ", 0

emptymess db "Oh no! STACK UNDERFLOW :( ", 0

fullmess db "Oh no! STACK OVERFLOW :( ", 0

disp1 db "Stack: [ ", 0

disp2 db ", ", 0

disp3 db " ]", 0

peek1 db "Peek: ", 0

.CODE

CreateStack Proc

mov eax, offset arr - 2

mov ebp, eax

;Message

mov edx, offset str1		; Move the address of the string into EDX

call WriteString			; Call the WriteString procedure to print the string

jmp TakeInput

AgainMessage:

mov edx, offset str2		; Move the address of the string into EDX

call WriteString			; Call the WriteString procedure to print the string

TakeInput:

mov eax, 0

mov ax, arrsize

call ReadInt

mov arrsize, ax

call CrLf

;----- Input Validation -----

cmp arrsize, bx

jl AgainMessage

cmp arrsize, cx

jg AgainMessage

ret

CreateStack Endp

IsFull Proc

mov dx, 0

mov eax, offset arr

mov ebx, 0

mov bx, arrsize

dec bx

add bx, bx

add eax, ebx

cmp ebp, eax

jne notfull

mov dx, 1

notfull:

ret

IsFull Endp

IsEmpty Proc

mov dx, 0

mov eax, offset arr

sub eax, 2

cmp ebp, eax

jne notempty

mov dx, 1

notempty:

ret

IsEmpty Endp

Pushst Proc

Call isFull

cmp dx, 1

je itsfull

add ebp, 2

mov dx, inputval

mov [ebp], dx

jmp ohh

itsfull:

Call CrLf

mov edx, offset fullmess

call WriteString

call CrLf

ohh:

ret

Pushst Endp

Popst Proc

Call isEmpty

cmp dx, 1

je itsempty

sub ebp, 2

jmp yaaay

itsempty:

Call CrLf

mov edx, offset emptymess

call WriteString

call CrLf

yaaay:

ret

Popst Endp

Peekst Proc

Call isEmpty

cmp dx, 1

je itsempty

mov edx, offset peek1

Call WriteString

mov eax, 0

mov ax, [ebp]

call WriteInt

call CrLf

jmp yaay

itsempty:

Call CrLf

mov edx, offset emptymess

call WriteString

call CrLf

yaay:

ret

Peekst Endp

Display Proc

mov edx, 0

mov eax, 0

mov edi, offset arr

Call CrLf

mov edx, offset disp1

Call WriteString

Call isEmpty

cmp dx, 1

je itsempty

mov eax, 0

PrintL:

mov ax, [edi]

Call WriteInt

cmp edi, ebp

je Exitploop

mov edx, offset disp2

Call WriteString

add edi, 2

jmp PrintL

Exitploop:

itsempty:

mov edx, offset disp3

Call WriteString

Call CrLf

ret

Display Endp

main Proc

mov bx, 0

mov cx, 100

Call CreateStack

mov bx, 3

mov inputval, bx

Call Pushst

mov bx, 5

mov inputval, bx

Call Pushst

mov bx, 7

mov inputval, bx

Call Pushst

mov bx, 6

mov inputval, bx

Call Pushst

mov bx, 4

mov inputval, bx

Call Pushst

mov bx, 2

mov inputval, bx

Call Pushst

Call Display

Call Popst

Call Popst

Call Popst

Call Popst

Call Popst

Call Popst

Call Display

exit						; Exit the program

main endp

end main

### 10. Reverse a String using Stack

- Problem Statement: Write a program to reverse a string using stack operations. Push each character onto the stack and then pop them in reverse order to form the reversed string.

- Input: "hello"

- Output: "ello"

Code:

include irvine32.inc

.DATA

arr db "hello", 0

arrs = lengthof arr - 1

.CODE

reverse Proc

mov ecx, arrs

mov esi, offset arr

mov eax, 0

L1:

mov al, byte ptr [esi]

push eax

inc esi

Loop L1

mov ecx, arrs

mov esi, offset arr

L2:

pop [esi]

inc esi

Loop L2

ret

reverse Endp

main Proc

call reverse

mov ecx, arrs

mov edi, 0

mov edx, offset arr		; Move the address of the string into EDX

call WriteString			; Call the WriteString procedure to print the string

call CrLf					; Move to the next line after printing

exit						; Exit the program

main endp

end main