Assignment No. 01

Computer Organization & Assembly Language (EE-2003)

Deadline: 18th September 2024

ID: 23i-0523                              NAME: Muhammad Moiz Ansari                               SECTION: F

Data Guide:

F1 = m	(6D)			L1 = a	(61)			R =   0523

F2 = o	(6F)			L2 = n	(6E)			R1 = 05

F3 = i	(69)			L3 = s	(73)			R2 = 23

F4 = z	(7A)			L4 = a	(61)			RR = 05230523

Space: 20h

- Consider the following Data declaration. Fill in the given Memory for following declaration. Look for errors if any.

ASM Program

.data valb_1 BYTE 'm' valb_2 BYTE 1111d ;error valb_3 DB +127 valb_3a DB 256 ;error valb_4 BYTE -128 valb_4a BYTE -129 valb_4b SBYTE -128 valb_5 BYTE 255 valb1_6 SBYTE -129 valb1_7 SBYTE +127 valb_8 SBYTE 'o' valb1_9 SBYTE -Ah .code main Proc mov edi,offset valb_1 mov al,valb_1 main ENDP END main

MEMORY

00; 01; 02; 03; 04; 05; 06; 07; 08; 09; 0A; 0B; 0C; 0D; 0E; 0F. 00404000.

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data DB -5 List1 DB 10, 041h ,'A', 00111111b List2 BYTE 1,2,3,4 BYTE 0101b,00010111b,5 BYTE 8,10,11,12 string BYTE 'ABC',0 string2 BYTE "abc",0 string3 BYTE "moiz 'INSIDE''singlequote",0 string7 db 'ansari "INSIDE " doublequote',0 string4 DB \ ;\ line continuation character "Hello" string5 b \ ;;;;;; yte "hello" string5 \ DB "Hello again" list3 db 1,2, \ 3,4 |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset List1 mov al,list1-1 mov al,List1+5 mov bl,string+4 mov cl,string2+3 mov dl,string3+5 mov ah,string4+1 mov bh,string5+4 mov ch,string7+3 main ENDP END main | EDI. AL. AL. BL. CL. DL. AH. BH. CH. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020. 00404030. 00404040. 00404050. 00404060. 00404070. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data list1 BYTE 1,0101b list2 BYTE 3 DUP(-2,00010111b ) list3 DB 2 DUP(?,1, 'm') list4 BYTE 3 DUP('AB') list5 BYTE 2 DUP(‘moiz’) |  |
| CODE | REGISTER |
| .code main Proc mov edi,offset list1 mov al, List2+2 mov bl, List3+4 mov cl, List4+3 mov dl, List5+3 main ENDP END main | EDI. AL. BL. CL. DL. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data word1 DW 1 word2 DW -1 word3 SWORD -1 word4 SWORD +1 word5 DW 'AB' word6 DW 'ABC' list1 DW 2 DUP(214h) list2 DW 1024o,2 list3 WORD 3 DUP(0) list4 WORD 2 DUP(0AB12H) List5 WORD 01F2BH,01101011001011b, 523d list6 DW 3 DUP('AB') |  |
| CODE | REGISTER |
| .code main Proc mov edi, offset list1 mov al, word1 mov bx, List1+4 mov cx, List2+1 mov dx, List4+2 mov ax, List6+1 mov cx, word5+2 mov cx, word5+4 main ENDP END main | EDI. AL. BX. CX. DX. AX. CX. CX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020. 00404030. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data dword1 DD 1 dword2 DD -1 dword3 DWORD -1 dword4 DWORD 65536 dword5 DD 'XY' dword6 DD 'EFGH' List1 DWORD 012345678H,0011010011101011001011b,65 list4 DWORD 2 DUP(0AB123456H) |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset dword1 mov ebx, dword2 mov ecx, dword5 mov edx, dword6 mov eax, List1+4 mov ebx, list4+2 mov ecx, List1+8 mov ecx, dword5+4 main ENDP END main | EDI. EBX. ECX. EDX. EAX. EBX. ECX. ECX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020. 00404030. |

- Consider the following Data declaration.  Fill in the given Memory for following declaration. Look for errors if any.

DATA

.data v db 0 tval1 TBYTE 01234H tval2 DT 80000000000000001234h tval3 TBYTE +1234 tval4 TBYTE 1 tval5 TBYTE -1 val1 db 0EEH

MEMORY

0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020. 00404030.

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data v3 db count count=2*2+4-2 v1 db count count= 00010111b v2 db count count2= 'A' v3 db count2 count2='ABCD' count3= 'moiza' ;5 characters from your name ;no space v4 dd count2 fcount=2.5 CountMax=4294967296 CountMax=4294967294 |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset v1 count = 5 mov ax,0 mov al,v1 mov bl,v2 mov cl,count count=257 mov cl,count mov cx,count main ENDP END main | EDI. AX. AL. BL. CL. CL. CX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data firstoffset dd $ list1 db -1,-2,-3, 'm' list1size= $-list1 list2 dw 1,2, 27o,4,5, 20Bh list2size= ($-list2)/2 list3 dd 00010111b,2,0Bh list3size= ($-list3)/4 |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset firstoffset mov ah,list1+3 mov bh,list1size mov cx,list2 mov dh,list2size mov dL,list3size mov esi,$ main ENDP END main | EDI. AH. BH. CX. DH. DL. ESI. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data v1 db 1 exp equ 2*5 exp=75 exp equ 9 txt equ <"ABC"> txt equ <"abc"> txt2 equ <10*10> symb equ exp mv equ mov axmov equ <mv al,v1> axmove equ <mov al,v1> VAL1 db 6 list db exp,txt,txt2,symb axmov equ <mv bl,v1> |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset VAL1 mov ax,0 mov al,v1 mov dx,txt2 axmov axmove axmov equ <mv cl,v1> axmov exp equ 255 mov BX,exp main ENDP END main | EDI. AX. AL. DX. BL. AL. CL. BX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data v1 db 1 exp textequ %(2*5) exp textequ %(20) txt textequ <"Message"> txt2 textequ txt move textequ <MOV> axmov textequ < move al,v1> msg db txt,txt2 |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset v1 mov al,v1 axmov mov al,exp exp textequ %(30) axmov textequ < move bl,v1> axmov mov Cl,exp main ENDP END main | EDI. AL. AL. AL. BL. CL. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data val1 db 1001b val2 db -10010011y val3 dw 1.0 val4 db -34o val5 dw 5732q val6 db 2.5 val7 db 1+'a'+4/2 val8 dw 5*3/4*2+8 val9 db -255 val4 dd 1.0 wval WORD 0 result word ? |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset val1 mov ax,3.5 mov bl,val2*2 mov al,2*(128+1) mov cl, val3-1 mov cl,val1+val1a mov al,-45o mov ax,'a' mov ax,'ab' mov ax,AB12h mov al,5*(-3) mov al,-(5*3) mov al,(-3)5 mov al,val3 mov bx,val1 add bx,al mov result,bx mov result,bl add val1,val2 mov 3,val2 main ENDP END main | EDI. AX. BL. AL. CL. CL. AL. AX. AX. AX. AL. AL. AL. AL. BX. BX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 BYTE 16 bv2 BYTE -16 wval WORD 0 |  |
| CODE | REGISTER |
| .code main Proc movsx bx,al movsx cx,bv2 movsx eax,bv2 movsx ebx,bval1 movzx ecx,bv2 movzx edx,bval1 main ENDP END main | BX. CX. EAX. EBX. ECX. EDX. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 BYTE 5 bval2 BYTE 7 wval1 WORD 0ABCDH wval2 word 01234h |  |
| CODE | REGISTER |
| .code main Proc mov al,bval1 mov bl,bval2 xchg al,bl ; mov bval1,bl mov bval2,al mov cx,wval1 xchg cx,wval2 mov wval2,cx main ENDP END main | AL. BL. AL. CX. CX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bary BYTE 0ABh,045O, 17h,25T,50q wary WORD 3456O,20Bh ,'cd' Dary dd 1776403o,'ABcd','ansa',0AB12CD34h Qary dq 12345678abcdefh,'AbCd' |  |
| CODE | REGISTER |
| .code main Proc mov al,bary mov bl,bary+1 mov cl,bary+2 mov dl,bary+3 mov ax,wary mov bx,wary+2 mov cx,wary+6 mov dx,wary+12 mov eax,dary+4 mov ebx,wary+8 mov edx,wary main ENDP END main | AL. BL. CL. DL. AX. BX. CX. DX. EAX. EBX. EDX. |

- Consider the following Data declaration.  Fill in the given Memory for following declaration. Look for errors if any.

DATA

.data bval1 DB 1,2, 23d Align 2 wval2 DW 0010 0000 1011b bval2 db 05d Align dword dval3 DD 7FD03h Align 4 val4 DB 'm' ALIGN WORD val5 DB 'a' pary DD bval1

MEMORY

0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020.

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 DB 034h, 0101b wval2 DW 20Bh dval3 DD 0111 1111 1101 0000 0011b |  |
| CODE | REGISTER |
| .code main Proc mov esi,OFFSET bval1 mov ax, WORD PTR bval1 mov al, BYTE PTR wval2 mov bx, WORD PTR dval3 mov cx, WORD PTR [dval3+2] main ENDP END main | ESI. AX. AL. BX. CX. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 DB 'm' wval2 DW 0ABCDH,'o' dval3 DD 07FD03h qval4 QWORD 01243567812435678H tval5 tbyte -1 |  |
| CODE | REGISTER |
| .code main Proc mov al, TYPE bval1 mov bl, TYPE wval2 mov cl, TYPE dval3 mov dl, TYPE qval4 mov ah, TYPE tval5 main ENDP END main | AL. BL. CL. DL. AH. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bAry BYTE 010H,020h,030H wAry WORD 5 DUP(?),0 ,0 string1 BYTE 0FFH,0FFH dAry DWORD 2 DUP(3 DUP(0)),0 string2 BYTE 'ABCDEFGHIJKLMNOPQRSTUVWXYZ',0 str2len= $-string2 bAry2 BYTE 1,2,3, 4,5,6 BYte 4,5,6 r dq 2 dup(5),2 dup('abcd') |  |
| CODE | REGISTER |
| .code main Proc mov bl, LENGTHOF bAry mov cl, LENGTHOF wAry mov dl, LENGTHOF dAry mov ah, LENGTHOF string2 mov bh ,str2len mov al, LENGTHOF bAry2 mov bl, LENGTHOF r main ENDP END main | BL. CL. DL. AH. BH. AL. BL. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bAry BYTE 010H,020h,030H wAry WORD 5 DUP(?),0 ,0 string1 BYTE 0FFH,0FFH dAry DWORD 2 DUP(3 DUP(0)),0 string2 BYTE 'ABCDEFGHIJKLMNOPQRSTUVWXYZ',0 r dq 2 dup(5),2 dup('abcd') |  |
| CODE | REGISTER |
| .code main Proc mov bl, SIZEOF bAry mov cl, SIZEOF wAry mov dl, SIZEOF dAry mov ah, SIZEOF string1 mov al, SIZEOF string2 mov bl, SIZEOF r main ENDP END main | BL. CL. DL. AH. AL. BL. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data valb LABEL Byte valw LABEL WORD vald LABEL DWORD valq LABEL dq valt Tbyte 12345678abcdef123456h |  |
| CODE | REGISTER |
| .code main Proc mov al,valb mov bx,valw mov ecx,vald main ENDP END main | AL. BX. ECX. |

- Consider the following Data declaration. Fill in the given Memory and register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data Ary1 BYTE 'm',29, 27o ary2 WORD 'a',05678h, 0010 0000 1011b |  |
| CODE | REGISTER |
| .code main Proc mov ESI, OFFSET Ary1 mov al,[ESI] add ESI,1 mov bl,[ESI] inc ESI inc BYTE PTR [ESI] mov cl,[ESI] mov ESI, OFFSET Ary2 mov ax,[ESI] add ESI,2 mov bx,[ESI] inc ESI inc ESI inc WORD PTR [ESI] mov cx,[ESI] main ENDP END main | ESI. AL. ESI. BL. ESI. CL. ESI. AX. ESI. BX. ESI. CX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000. 00404010. 00404020. 00404030. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data Ary1 BYTE 'm',2, 17h,4 ary2 WORD 'a',05678h, 1024o,567o ary3 DWORD 'n',012345678h, 07FD03h,567o |  |
| CODE | REGISTER |
| .code main Proc mov ESI,0 mov al,ary1[ESI] mov ESI,1 * TYPE ary1 mov bl,ary1[ESI] mov ESI,2 * TYPE ary1 mov cl,ary1[ESI] mov ESI,0 mov ax,ary2[ESI] mov ESI,1 * TYPE ary2 mov bx,ary2[ESI] mov ESI,2 * TYPE ary2 mov cx,ary2[ESI] mov ESI,0 mov eax,ary3[ESI] mov ESI,1 * TYPE ary3 mov ebx,ary3[ESI] mov ESI,2 * TYPE ary3 mov ecx,ary3[ESI] main ENDP END main | ESI. AL. ESI. BL. ESI. CL. ESI. AX. ESI. BX. ESI. CX. ESI. EAX. ESI. EBX. ESI. ECX. |

- Consider the following Data declaration. Fill in the given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data ary WORD 'a',05678h, 0010 0000 1011b,567o |  |
| CODE | REGISTER |
| .code main Proc mov ESI,0 mov ax,ary[ESI * TYPE ary] INC ESI mov BX,ary[ESI * TYPE ary] INC ESI mov CX,ary[ESI * TYPE ary] INC ESI main ENDP END main | ESI. AX. ESI. BX. ESI. CX. ESI. |

- Write an assembly program that reverses the following string in the memory. Attach screenshot. Only ‘jmp’ and labels are allowed:

'!egaugnal ylbmessA eroda I'

- ❌  Implement following C code in assembly language. Attach screenshot. You can use jmp/labels and loop:

- Write code that initializes the following array with the first 100 integers using INDIRECT ADDRESSING. You must upload a screenshot of memory. Only ‘jmp’ and labels are allowed:

- Write an assembly program that converts your name from lowercase to uppercase. You must update the name in the memory and attach a screenshot of it. You can use jmp/labels or loop.

- ❌  Write an assembly language program that calculates factorial for any number. Attach screenshot as well. You can use jmp/labels or loop.