Assignment No. 01 Solution

Computer Organization & Assembly Language (EE-2003)

Read the Instructions Carefully

- Assignments are designed for your learning copying will lead to plagiarism. Practicing the questions yourself will help you in your exams. Fill the memory and register tables in the word file first before running the respective programs in visual studio.

- The filled memory should represent the memory status of the executed program.

- Memory and register values should be in hexadecimal format.

- Each row in the memory table consists of 16 columns, each of 1 byte. The row indicates the starting offset. Both the starting offset and the columns are represented with hexadecimal values.

- Your assignment marks will be calculated from your submission (60%) + Quiz (30%) + Sessional-I (10%)

- You are supposed to debug each question in Visual Studio after filling the tables.

- You are supposed to submit a SNAPSHOT of your visual studio debugging screen, such that it shows the status of registers and memory after debugging. Your submission must include snap shots of each question with a visible time and date of your computer. The snapshots for each question must be attached in the word file under respective questions.

- Submit a PDF File with the naming format 23i-1234_Section_A1

- If you find any ERROR in the program highlight the error and write the justification for the error. You must understand ERROR carefully. After identifying the error, you will ignore it by commenting it out and running the program. Also, the register values corresponding to the erroneous statements must be left blank. For example:

; mov al, 256 ERROR: 256 is out of range for an 8-bit register like `AL`. It should be within 0-255.

- Data Declarations highlighted in yellow must be replaced as instructed.

- ‘F1’ represents the first character of your first name, ‘F2’ represents the second character, ‘F3’ and ‘F4’ represent the third and the fourth characters of your first name, respectively.  If your first name has less characters, then repeat the previous character(s) as needed.

- ‘L1’ represents the first character of your last name, ‘L2’ represents the second character, ‘L3’ and ‘L4’ represent the third and the fourth characters of your last name, respectively. If your last name has less characters, then repeat the previous character(s) as needed.

- ‘R’ represents the numeric part of your roll number. For example, if your roll number is 23i-1234, then ‘R’ is 1234. The base for ‘R’ is represented as (base), where the base can be ‘h’ for hexadecimal, ‘d’ for decimal, ‘b’ for binary, and ‘o’ for octal. You must first treat ‘R’ as a decimal number, then convert it into the specified base before substituting the value.

When R is 1234, then R(d) = 1234d

R(h) = 04D2h

R(o) = 2322o

R(b) = 0100 1101 0010b

- ‘R1’ and ‘R2’ represent first and second part of ‘R’, respectively. For example, if ‘R’ is 1234, then ‘R1’ is 12 and ‘R2’ is 34. The base for ‘R1’ and ‘R2’ are represented as (base), where the base can be ‘h’ for hexadecimal, ‘d’ for decimal, ‘b’ for binary, and ‘o’ for

octal. You must first treat ‘R1’ and ‘R2’ as decimal numbers, then convert them into the specified bases before substituting the values.

- ‘RR’ represents the concatenated roll number. For example, if your roll number is 23i-1234 then ‘RR’ is 12341234. The “base” rules defined above also apply here.

Solution for roll number, 1234 and name

“Ahmed Ali”

- R(d) = 1234d

- R(h) = 04D2h

- R(o) = 2322o

- R(b) = 0100 1101 0010b

- R1(d) = 12d

- R1(h) = 0Ch

- R1(o) = 14o

- R1(b) = 1100b

- R2(d) = 34d

- R2(h) = 022h

- R2(o) = 42o

- R2(b) = 0010 0010b

- RR(d) = 12341234d

- RR(h) = 0BC4FF2h

- RR(o) = 57047762o

- RR(b) = 1011 1100 0100 1111 1111 0010b

- First Name = Ahmed (F1 = A, F2 = h, F3 = m, F4 = e)

- Last Name = Ali (L1 = A, L2 = l, L3 = i, L4 = i)

- Consider the following Data declaration?  Fill given Memory for following declaration. Look for errors if any.

ASM Program

.data valb_1 BYTE 'A' ;valb_2 BYTE 1111d Error: Out of range 1111 > 255, initializer must be in the range 0 to 255 valb_3 DB +127 ; valb_3a DB 256 Error: Out of range 256 > 255, initializer must be in the range 0 to 255 valb_4 BYTE -128 valb_4a BYTE -129 valb_4b SBYTE -128 valb_5 BYTE 255 valb1_6 SBYTE -129 valb1_7 SBYTE +127 valb_8 SBYTE 'h' ;valb1_9 SBYTE -Ah Error: Ah is treated as a variable, write 0 before A for assembler to consider it as a constant hexadecimal expression .code main Proc mov edi,offset valb_1 mov al,valb_1 main ENDP END main

MEMORY

0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 41; 7f; 80; 7f; 80; Ff; 7f; 7f; 68; 00; 00; 00; 00; 00; 00; 00.

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors as well.

| DATA |  |
| --- | --- |
| .data DB -5 List1 DB 10, 041h ,'A', 00111111b List2 BYTE 1,2,3,4 BYTE 1100b, 00100010b,5 BYTE 8,10,11,12 string BYTE 'ABC',0 string2 BYTE "abc",0 string3 BYTE "Ahmed 'INSIDE''singlequote",0 string7 db 'Ali "INSIDE " doublequote',0 string4 DB \ ;\ line continuation character "Hello" ;string5 b \ yte "hello" Error: line continuation character is preceded by a space that’s why ‘byte’ becomes ‘b yte’ which is not a data type in asm string5 \ DB "Hello again" list3 db 1,2, \ 3,4 |  |
| CODE | REGISTER |
| .code main Proc mov edi,offset List1 mov al,list1-1 mov al,List1+5 mov bl,string+4 mov cl,string2+3 mov dl,string3+5 mov ah,string4+1 mov bh,string5+4 mov ch,string7+3 main ENDP END main | EDI; 00404001. AL; FB. AL; 02. BL; 61. CL; 00. DL; 20. AH; 65. BH; 6F. CH; 20. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; Fb; 0a; 41; 41; 3f; 01; 02; 03; 04; 0C; 22; 05; 08; 0a; 0b; 0c. 00404010; 41; 42; 43; 00; 61; 62; 63; 00; 41; 68; 6d; 65; 64; 20; 27; 49. 00404020; 4e; 53; 49; 44; 45; 27; 27; 73; 69; 6e; 67; 6c; 65; 71; 75; 6f. 00404030; 74; 65; 00; 41; 6c; 69; 20; 22; 49; 4e; 53; 49; 44; 45; 20; 20. 00404040; 22; 20; 64; 6f; 75; 62; 6c; 65; 71; 75; 6f; 74; 65; 00; 48; 65. 00404050; 6c; 6c; 6f; 48; 65; 6c; 6c; 6f; 20; 61; 67; 61; 69; 6e; 01; 02. 00404060; 03; 04; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. 00404070; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data list1 BYTE 1,1100b list2 BYTE 3 DUP(-2,00100010b) list3 DB 2 DUP(?,1, 'A') list4 BYTE 3 DUP('AB') list5 BYTE 2 DUP('Ahmed') |  |
| CODE | REGISTER |
| .code main Proc mov edi,offset list1 mov al, List2+2 mov bl, List3+4 mov cl, List4+3 mov dl, List5+3 main ENDP END main | EDI; 00404000. AL; FE. BL; 01. CL; 42. DL; 65. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 01; 0C; Fe; 22; Fe; 22; Fe; 22; 00; 1; 41; 00; 1; 41; 41; 42. 00404010; 41; 42; 41; 42; 41; 68; 6d; 65; 64; 41; 68; 6d; 65; 64; 00; 00. 00404020; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data word1 DW 1 word2 DW -1 word3 SWORD -1 word4 SWORD +1 word5 DW 'AB' ;word6 DW 'ABC' Error: ‘ABC’ takes 3 bytes can’t be stored in the identifier of size 2 bytes list1 DW 2 DUP(04D2h) list2 DW 2322o,2 list3 WORD 3 DUP(0) list4 WORD 2 DUP(0AB12H) List5 WORD 01F2BH,01101011001011b, 1234d list6 DW 3 DUP('AB') |  |
| CODE | REGISTER |
| .code main Proc mov edi,offset list1 ;mov al, word1 Error: al register can store maximum 8 bits (1 byte), word1 is 16 bits (2 bytes) mov bx, List1+4 mov cx, List2+1 mov dx, List4+2 mov ax, List6+1 mov cx, word5+2 mov cx, word5+4 main ENDP END main | EDI; 0040400A. AL. BX; 04D2. CX; 0204. DX; AB12. AX; 4241. CX; 04D2. CX; 04D2. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 01; 00; Ff; Ff; Ff; Ff; 01; 00; 42; 41; D2; 04; D2; 04; D2; 04. 00404010; 02; 00; 00; 00; 00; 00; 00; 00; 12; Ab; 12; Ab; 2b; 1f; Cb; 1a. 00404020; D2; 04; 42; 41; 42; 41; 42; 41; 00; 00; 00; 00; 00; 00; 00; 00. 00404030; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data dword1 DD 1 dword2 DD -1 dword3 DWORD -1 dword4 DWORD 65536 dword5 DD 'XY' dword6 DD 'EFGH' List1 DWORD 012345678H,0011010011101011001011b,65 list4 DWORD 2 DUP(0AB123456H) |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset dword1 mov ebx, dword2 mov ecx, dword5 mov edx, dword6 mov eax, List1+4 mov ebx, list4+2 mov ecx, List1+8 mov ecx, dword5+4 main ENDP END main | EDI; 00404000. EBX; FFFFFFFF. ECX; 00005859. EDX; 45464748. EAX; 000D3ACB. EBX; 3456AB12. ECX; 00000041. ECX; 45464748. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 01; 00; 00; 00; Ff; Ff; Ff; Ff; Ff; Ff; Ff; Ff; 00; 00; 01; 00. 00404010; 59; 58; 00; 00; 48; 47; 46; 45; 78; 56; 34; 12; Cb; 3a; 0d; 00. 00404020; 41; 00; 00; 00; 56; 34; 12; Ab; 56; 34; 12; Ab; 00; 00; 00; 00. 00404030; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

DATA

.data v db 0 tval1 TBYTE 01234H tval2 DT 80000000000000001234h tval3 TBYTE +1234 tval4 TBYTE 1 tval5 TBYTE -1 val1 db 0EEH

MEMORY

0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 00; 34; 12; 00; 00; 00; 00; 00; 00; 00; 00; 34; 12; 00; 00; 00. 00404010; 00; 00; 00; 00; 80; D2; 04; 00; 00; 00; 00; 00; 00; 00; 00; 01. 00404020; 00; 00; 00; 00; 00; 00; 00; 00; 00; Ff; Ff; Ff; Ff; Ff; Ff; Ff. 00404030; Ff; Ff; Ff; Ee; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00.

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data ; v3 db count Error count not declared count=2*2+4-2 v1 db count count= 00100010b v2 db count count2= 'A' ;v3 db count2 ; Error: v3 has already been defined above Redefinition not allowed count2='ABCD' ;count3= 'AhmeA' ;5 characters from your name no space ; Error maximum of 4 bytes can be stored in constant v4 dd count2 ; fcount=2.5 ; Error floating point not allowed ; CountMax=4294967296 Error value is exceeding the maximum size capacity of a constant that is 32 bits (4 bytes) CountMax=4294967294 |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset v1 count = 5 mov ax,0 mov al,v1 mov bl,v2 mov cl,count count=257 ;mov cl,count ;Error (sizemismatch) count takes 2 bytes count can’t be stored in the register of size 1 byte mov cx,count main ENDP END main | EDI; 00404000. AX; 0000. AL; 06. BL; 22. CL; 05. CL. CX; 0101. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 06; 22; 44; 43; 42; 41; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. 00404010; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data firstoffset dd $ list1 db -1,-2,-3, 'A' list1size= $-list1 list2 dw 1,2, 42o,4,5, 04D2h list2size= ($-list2)/2 list3 dd 00100010b,2,0Bh list3size= ($-list3)/4 |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset firstoffset mov ah,list1+3 mov bh,list1size mov cx,list2 mov dh,list2size mov dL,list3size mov esi,$ main ENDP END main | EDI; 00404000. AH; 41. BH; 04. CX; 0001. DH; 06. DL; 03. ESI; 00401028. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 00; 40; 40; 00; Ff; Fe; Fd; 41; 01; 00; 02; 00; 22; 00; 04; 00. 00404010; 05; 00; D2; 04; 22; 00; 00; 00; 02; 00; 00; 00; 0b; 00; 00; 00. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data v1 db 1 exp equ 2*5 ; exp=75 Error: equ type literal can’t be redefined ; exp equ 9 Error: equ type literal can’t be redefined txt equ <"ABC"> txt equ <"abc"> txt2 equ <10*10> symb equ exp mv equ mov axmov equ <mv al,v1> axmove equ <mov al,v1> VAL1 db 6 list db exp,txt,txt2,symb axmov equ <mv bl,v1> |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset VAL1 mov ax,0 mov al,v1 mov dx,txt2 axmov axmove axmov equ <mv cl,v1> axmov ; exp equ 255 Error: equ type literal can’t be redefined mov BX,exp main ENDP END main | EDI; 00404001. AX; 0000. AL; 01. DX; 0064. BL; 01. AL; 01. CL; 01. BX; 000A. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 01; 06; 0a; 61; 62; 63; 64; 0a; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given register values? Look for errors if any.

| DATA |  |
| --- | --- |
| .data v1 db 1 exp textequ %(2*5) exp textequ %(20) txt textequ <"Message"> txt2 textequ txt move textequ <MOV> axmov textequ < move al,v1> msg db txt,txt2 |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset v1 mov al,v1 axmov mov al,exp exp textequ %(30) axmov textequ < move bl,v1> axmov mov Cl,exp main ENDP END main | EDI; 00404000. AL; 01. AL; 01. AL; 14. BL; 01. CL; 1E. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data val1 db 1001b val2 db -10010011y val3 dw 1.0 val4 db -34o val5 dw 5732q val6 db 2.5 val7 db 1+'a'+4/2 val8 dw 5*3/4*2+8 val9 db -255 ; val4 dd 1.0 Error: redefining val4 wval WORD 0 result word ? |  |
| CODE | REGISTER |
| .code main Proc mov edi,Offset val1 ; mov ax,3.5 Error floating point not allowed For registers ; mov bl,val2*2 Error can’t multiply a memory variable, for this operation use ‘mul’ ; mov al,2*(128+1) Error complex arithmetic expressions aren’t supported by masm ; mov cl, val3-1 Error both operands must be of the same size ; mov cl,val1+val1a Error val1a not defined mov al,-45o mov ax,'a' mov ax,'ab' ; mov ax,AB12h Error asm is treating AB12h as a variable, instead write 0AB12h for a hexadecimal value mov al,5*(-3) mov al,-(5*3) ; mov al,(-3)5 Error operator missing ; mov al,val3 Error both operands must be of the same size ; mov bx,val1 Error both operands must be of the same size ; add bx,al Error adding 1 byte register in 2 byte, size of the registers must be same for addition mov result,bx ; mov result,bl Error both operands must be of the same size ; add val1,val2 Error memory to memory not allowed when using add instruction ; mov 3,val2 Error lvalue must be a constant literal or a register or a variable main ENDP END main | EDI; 00404000. AX. BL. AL. CL. CL. AL; DB. AX; 0061. AX; 6162. AX. AL; F1. AL; F1. AL. AL. BX. BX. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 09; 6d; 00; 00; E4; Da; 0b; 00; 64; 0e; 00; 01; 00; 00; 00; D0. 00404010; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given register values? Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 BYTE 16 bv2 BYTE -16 wval WORD 0 |  |
| CODE | REGISTER |
| .code main Proc movsx bx,al movsx cx,bv2 movsx eax,bv2 movsx ebx,bval1 movzx ecx,bv2 movzx edx,bval1 main ENDP END main | BX; FFCC. CX; FFF0. EAX; FFFFFFF0. EBX; 00000010. ECX; 000000F0. EDX; 00000010. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 BYTE 5 bval2 BYTE 7 wval1 WORD 0ABCDH wval2 word 01234h |  |
| CODE | REGISTER |
| .code main Proc mov al,bval1 mov bl,bval2 xchg al,bl ; mov bval1,bl mov bval2,al mov cx,wval1 xchg cx,wval2 mov wval2,cx main ENDP END main | AL; 05. BL; 07. AL; 07. CX; ABCD. CX; 1234. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 05; 07; Cd; Ab; Cd; Ab; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration. Fill given register values. Look for errors if any.

| DATA |  |
| --- | --- |
| .data bary BYTE 0ABh,045O,22h,25T,50q wary WORD 3456O,04D2h ,'cd' Dary dd 57047762o,'ABcd','Alii',0AB12CD34h Qary dq 12345678abcdefh,'AbCd' |  |
| CODE | REGISTER |
| .code main Proc mov al,bary mov bl,bary+1 mov cl,bary+2 mov dl,bary+3 mov ax,wary mov bx,wary+2 mov cx,wary+6 mov dx,wary+12 mov eax,dary+4 ; mov ebx,wary+8 Error both operands must be of the same size ; mov edx,wary Error both operands must be of the same size main ENDP END main | AL; AB. BL; 25. CL; 22. DL; 19. AX; 072E. BX; 04D2. CX; 4FF2. DX; 4142. EAX; 41426364. EBX. EDX. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

DATA

.data bval1 DB 1,50q, 34d Align 2 wval2 DW 010011010010b bval2 db 12d Align dword dval3 DD 0BC4FF2h Align 4 val4 DB 'A' ALIGN WORD val5 DB 'A' pary DD bval1

MEMORY

0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 01; 28; 22; 00; D2; 04; 0c; 00; F2; 4f; Bc; 00; 41; 00; 41; 00. 00404010; 40; 40; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00; 00.

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 DB 034h, 1100b wval2 DW 04D2h dval3 DD 101111000100111111110010b |  |
| CODE | REGISTER |
| .code main Proc mov esi,OFFSET bval1 mov ax, WORD PTR bval1 mov al, BYTE PTR wval2 mov bx, WORD PTR dval3 mov cx, WORD PTR [dval3+2] main ENDP END main | ESI; 00404000. AX; 0C34. AL; D2. BX; 4FF2. CX; 00BC. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data bval1 DB 'A' wval2 DW 0ABCDH, 'l' dval3 DD 0BC4FF2h qval4 QWORD 01243567812435678H tval5 tbyte -1 |  |
| CODE | REGISTER |
| .code main Proc mov al, TYPE bval1 mov bl, TYPE wval2 mov cl, TYPE dval3 mov dl, TYPE qval4 mov ah, TYPE tval5 main ENDP END main | AL; 01. BL; 02. CL; 04. DL; 08. AH; 0A. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data bAry BYTE 010H,020h,030H wAry WORD 5 DUP(?),0 ,0 string1 BYTE 0FFH,0FFH dAry DWORD 2 DUP(3 DUP(0)),0 string2 BYTE 'ABCDEFGHIJKLMNOPQRSTUVWXYZ',0 str2len= $-string2 bAry2 BYTE 1,2,3, 4,5,6 BYte 4,5,6 r dq 2 dup(5),2 dup('abcd') |  |
| CODE | REGISTER |
| .code main Proc mov bl, LENGTHOF bAry mov cl, LENGTHOF wAry mov dl, LENGTHOF dAry mov ah, LENGTHOF string2 mov bh ,str2len mov al, LENGTHOF bAry2 mov bl, LENGTHOF r main ENDP END main | BL; 03. CL; 07. DL; 07. AH; 1B. BH; 1B. AL; 06. BL; 04. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data bAry BYTE 010H,020h,030H wAry WORD 5 DUP(?),0 ,0 string1 BYTE 0FFH,0FFH dAry DWORD 2 DUP(3 DUP(0)),0 string2 BYTE 'ABCDEFGHIJKLMNOPQRSTUVWXYZ',0 r dq 2 dup(5),2 dup('abcd') |  |
| CODE | REGISTER |
| .code main Proc mov bl, SIZEOF bAry mov cl, SIZEOF wAry mov dl, SIZEOF dAry mov ah, SIZEOF string1 mov al, SIZEOF string2 mov bl, SIZEOF r main ENDP END main | BL; 03. CL; 0E. DL; 1C. AH; 02. AL; 1B. BL; 20. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data valb LABEL Byte valw LABEL WORD vald LABEL DWORD ; valq LABEL dq Error syntax error use Qword instead of dq valt Tbyte 12345678abcdef123456h |  |
| CODE | REGISTER |
| .code main Proc mov al,valb mov bx,valw mov ecx,vald main ENDP END main | AL; 56. BX; 3456. ECX; EF123456. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data Ary1 BYTE 'A',29, 42o ary2 WORD 'A',05678h, 010011010010b |  |
| CODE | REGISTER |
| .code main Proc mov ESI, OFFSET Ary1 mov al,[ESI] add ESI,1 mov bl,[ESI] inc ESI inc BYTE PTR [ESI] mov cl,[ESI] mov ESI, OFFSET Ary2 mov ax,[ESI] add ESI,2 mov bx,[ESI] inc ESI inc ESI inc WORD PTR [ESI] mov cx,[ESI] main ENDP END main | ESI; 00404000. AL; 41. ESI; 00404001. BL; 1d. ESI; 00404002. CL; 22+1=23. ESI; 00404003. AX; 0041. ESI; 00404005. BX; 5678. ESI; 00404007. CX; 04D2+1=04D3. |
| MEMORY |  |
| 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 41; 1d; 23; 41; 00; 78; 56; D3; 04; 00; 00; 00; 00; 00; 00; 00. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data Ary1 BYTE 'A',2, 022h,4 ary2 WORD 'A',05678h, 42o,567o ary3 DWORD 'l',012345678h, 0BC4FF2h,567o |  |
| CODE | REGISTER |
| .code main Proc mov ESI,0 mov al,ary1[ESI] mov ESI,1 * TYPE ary1 mov bl,ary1[ESI] mov ESI,2 * TYPE ary1 mov cl,ary1[ESI] mov ESI,0 mov ax,ary2[ESI] mov ESI,1 * TYPE ary2 mov bx,ary2[ESI] mov ESI,2 * TYPE ary2 mov cx,ary2[ESI] mov ESI,0 mov eax,ary3[ESI] mov ESI,1 * TYPE ary3 mov ebx,ary3[ESI] mov ESI,2 * TYPE ary3 mov ecx,ary3[ESI] main ENDP END main | ESI; 00000000. AL; 41. ESI; 00000001. BL; 02. ESI; 00000002. CL; 22. ESI; 00000000. AX; 0041. ESI; 00000002. BX; 5678. ESI; 00000004. CX; 0022. ESI; 00000000. EAX; 0000006C. ESI; 00000004. EBX; 12345678. ESI; 00000008. ECX; 00BC4FF2. |

- Consider the following Data declaration?  Fill given Memory and register value? Look for errors if any.

| DATA |  |
| --- | --- |
| .data ary WORD 'A',05678h, 010011010010b,567o |  |
| CODE | REGISTER |
| .code main Proc mov ESI,0 mov ax,ary[ESI * TYPE ary] INC ESI mov BX,ary[ESI * TYPE ary] INC ESI mov CX,ary[ESI * TYPE ary] INC ESI main ENDP END main | ESI; 00000000. AX; 0041. ESI; 00000001. BX; 5678. ESI; 00000002. CX; 04D2. ESI; 00000003. |

- Write an assembly program that reverses the following string in the memory. Upload memory screenshot as well after reversing the string .Only ‘jmp’ and labels are allowed:

'!egaugnal ylbmessA eroda I'

.386

.model flat,stdcall

.stack 4096

ExitProcess proto,dwExitCode:dword

.data

string db '!egaugnal ylbmessA eroda I',0

.code

main proc

mov eax,0

mov al,lengthof string

mov esi, offset string ; starting offset

mov edx, offset string

add edx,eax

sub edx,2	; ending offset

l1:

mov bl, [esi]

mov bh, [edx]

mov [esi], bh

mov [edx], bl

inc esi

dec edx

cmp esi, edx

jle l1 ; when both addresses will cross eachother donot jump to l1

invoke ExitProcess,0

main endp

end main

Memory View After running the program:

- Implement following C code in assembly language. You can use jmp/labels or loop.

.386

.model flat,stdcall

.stack 4096

ExitProcess proto,dwExitCode:dword

.data

.code

main proc

mov esi,0

mov ecx,3

outerloop:

mov ebx, ecx

sub ebx,3

innerloop:

cmp ebx,8

jg endinnerloop

inc esi

add ebx,2

jmp innerloop

endinnerloop:

loop outerloop

invoke ExitProcess,0

main endp

end main

- Write code that initializes the following array with first 100 integers using INDIRECT ADDRESSING. Only ‘jmp’ and labels are allowed:

.386

.model flat,stdcall

.stack 4096

ExitProcess proto,dwExitCode:dword

.data

ary dword 100 dup(0abcdef12h)

.code

main proc

mov esi, offset ary

mov eax, 1

mov ecx, lengthof ary

l1:

mov [esi],eax

add esi,type ary

inc eax

dec ecx

cmp ecx,0

jg l1

invoke ExitProcess,0

main endp

end main

- Write an assembly program that converts your name from lowercase to uppercase. You must update the name in the memory and attach a screenshot of it. You can use jmp/labels or loop.

.386

.model flat,stdcall

.stack 4096

ExitProcess proto,dwExitCode:dword

.data

string db "ali ahmed",0

.code

main proc

mov esi, offset string

mov ecx, lengthof string

sub ecx,1

mov al,32 ; to convert into uppercase

l1:

sub [esi], al

inc esi

loop l1

invoke ExitProcess,0

main endp

end main

- Write an assembly language program that calculates factorial of any number. You can use jmp/labels or loop.

.386

.model flat,stdcall

.stack 4096

ExitProcess proto,dwExitCode:dword

.data

num db 5	; factorial of num

result dw 1 ; to store factorial

.code

main proc

mov cl,num

l1:

movzx ax, cl

mul result

mov result,ax

sub cl,1

cmp cl,1

jg l1

invoke ExitProcess,0

main endp

end main