Assignment No. 02

Computer Organization & Assembly Language (EE-2003)

ID: __________                                      NAME: __________________                        SECTION: ____

Total Marks = 160

Read the Instructions Carefully

- Assignments are designed for your learning copying will lead to plagiarism. Practicing the questions yourself will help you in your exams. Fill the memory and register tables in the word file first before running the respective programs in visual studio.

- The filled memory should represent the memory status of the executed program.

- Memory and register values should be in hexadecimal format.

- Each row in the memory table consists of 16 columns, each of 1 byte. The row indicates the starting offset. Both the starting offset and the columns are represented with hexadecimal values.

- Your assignment marks will be calculated from your submission (60%) + Quiz (30%) + Sessional-I (10%)

- You are supposed to debug each question in Visual Studio after filling the tables.

- You are supposed to submit a SNAPSHOT of your visual studio debugging screen, such that it shows the status of registers and memory after debugging. Your submission must include snap shots of each question with a visible time and date of your computer. The snapshots for each question must be attached in the word file under respective questions. Remember each snapshot has marks.

- Submit a PDF File with the naming format 23i-1234_Section_CS

- If you find any ERROR in the program highlight the error and write the justification for the error. You must understand ERROR carefully. After identifying the error, you will ignore it by commenting it out and running the program. Also, the register values corresponding to the erroneous statements must be left blank. For example:

; mov al, 256 ERROR: 256 is out of range for an 8-bit register like `AL`. It should be within 0-255.

- Data Declarations highlighted in yellow must be replaced as instructed below:

- ‘id1’ represents the first character of your ID, ‘id2’ represents the second character, ‘id3’ and ‘id4’ represent the third and the fourth characters of your ID, respectively.

- Write equivalent C code for the assembly code given below? Update final value of registers in HEX

Int ax = 0;

for(int i = 5; i>0; i++)

{

for(int j = 3; j>0; j++)

{

ax+=1;

}

}

ASM Program

.data temp db 0 .code main Proc mov ax,0 mov ecx,5 Outer: mov temp,cl mov ecx,3 Inner: add ax,1 Loop Inner mov cx,0 mov cl,temp Loop Outer main ENDP END main

| AX | 0Fh |  |
| --- | --- | --- |
| CX | 00h |  |

- Update Flags after executing following code?

NOTE: AND performs a Boolean(bitwise) AND operation between each pair of matching bits in two operands and place result in the destination. AND instruction always clear overflow and carry flags. It modifies Sign, Zero and Parity flags.

| Flags | Sign | 1 |
| --- | --- | --- |
|  | Zero | 0 |
|  | Carry | 0 |
|  | Overflow | 0 |
|  | Parity | 1 |
|  | Auxiliary | 0 |

- Update memory after executing code given below

ASM Program

.data ary db 26 dup(?) arysize = $ - ary ary_copy db arysize dup(0) endmem db 1 .code main Proc mov esi, OFFSET ary mov edi, OFFSET ary_copy mov ax, 000FFh mov ecx, arysize and al, 061h mov bl, al and bl, 0CFh ;masking L1: mov [esi], al mov [edi], bl inc esi inc edi inc al inc bl Loop L1 main ENDP END main

MEMORY

0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 61h; 62h; 63h; 64h; 65h; 66h; 67h; 68h; 69h; 6Ah; 6Bh; 6Ch; 6Dh; 6Eh; 6Fh; 70h. 00404010; 71h; 72h; 73h; 74h; 75h; 76h; 77h; 78h; 79h; 7Ah; 41h; 42h; 43h; 44h; 45h; 46h. 00404020; 47h; 48h; 49h; 4Ah; 4Bh; 4Ch; 4Dh; 4Eh; 4Fh; 50h; 51h; 52h; 53h; 54h; 55h; 56h. 00404030; 57h; 58h; 59h; 5Ah; 01.

- Update Flags after executing following code

| Flags | Sign | 1 |
| --- | --- | --- |
|  | Zero | 0 |
|  | Carry | 0 |
|  | Overflow | 0 |
|  | Parity | 1 |
|  | Auxiliary | 0 |

NOTE: OR performs a Boolean(bitwise) OR operation between each pair of matching bits in two operands and place result in the destination. OR instruction always clear overflow and carry flags. It modifies Sign, Zero and Parity flags.

- Update memory after executing code given below

ASM Program

.data ary db 26 dup(?) arysize = $ - ary ary_copy db arysize dup(0) endmem db 1 .code main Proc mov esi, OFFSET ary mov edi, OFFSET ary_copy mov ax, 0041h mov ecx, arysize mov bl, al or bl, 00100000b L1: mov [esi], al mov [edi], bl inc esi inc edi inc al inc bl Loop L1 main ENDP END main

MEMORY

0; 1; 2; 3; 4; 5; 6; 7; 8; 9; A; B; C; D; E; F. 00404000; 41h; 42h; 43h; 44h; 45h; 46h; 47h; 48h; 49h; 4Ah; 4Bh; 4Ch; 4Dh; 4Eh; 4Fh; 50h. 00404010; 51h; 52h; 53h; 54h; 55h; 56h; 57h; 58h; 59h; 5Ah; 61h; 62h; 63h; 64h; 65h; 66h. 0404020; 67h; 68h; 69h; 6Ah; 6Bh; 6Ch; 6Dh; 6Eh; 6Fh; 70h; 71h; 72h; 73h; 74h; 75h; 76h. 00404030; 77h; 78h; 79h; 7Ah; 01.

| Flags | Sign | 0 |
| --- | --- | --- |
|  | Zero | 0 |
|  | Carry | 0 |
|  | Overflow | 0 |
|  | Parity | 1 |
|  | Auxiliary | 0 |

| AX | A5 | 93 |
| --- | --- | --- |
| AX | 5A | 6C |
| AX | 5A | 6C |
| AX | A5 | 93 |
| AX | 00 | 00 |
| AX | 5A | 37 |
| Al | 37 |  |
| AH | 5B |  |
| AL | 6C |  |

- Update register after each line of code and update Flags after executing XOR AL,AH

- Write a program that finds parity of number given below? HINT: Use XOR and LOOP to find parity

CODE:

mov eax, dword [parity]        ; Load the lower 32 bits into EAX

and eax, 0xFF                          ;Mask to isolate the lower 8 bits (only keep the last 8 bits)

xor ebx, ebx                            ; Clear EBX to store the parity (initially 0)

mov ecx, 8                               ; Set counter to 8 (for the lower 8 bits)

check_parity_loop:

and eax, 1                           ; Check the least significant bit

xor ebx, eax                       ; XOR parity with the LSB (toggle if the bit is 1)

shr eax, 1                           ; Shift right by 1 to check the next bit

loop check_parity_loop

INSTRUCTIONS: (Question 8-11) Perform each of the following operations on word size 2’s complement numbers and update the answer and value of flags after performing the arithmetic. Perform all the steps in the “calculation” box, only filling the answer will not get any credit.

- Update flags after arithmetic instruction? Also state which of the following jumps will taken or not taken

.code

mov ax,0FFFEh

add ax,0FC70h

jc l1

L1: jz L2

L2: jo L3

L3: js L4

L4: jp L5

L5:

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| Jc | 1 |  |
| Jz |  | 1 |
| Jo |  | 1 |
| Js | 1 |  |
| jp |  | 1 |

| Flags | Sign | 1 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 0 |  | 1 1 1 1 1 1 1 1 1 1 1 1111 1111 1111 1110 + 1 1111 1100 0111 0000 1111 1100 0110 1110 F C 6 E |
|  | Carry | 1 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 0 |  |  |

Update flags after arithmetic instruction? Also state which of the following jumps will taken or not taken

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| Jc | 1 |  |
| Jz |  | 1 |
| Jo | 1 |  |
| Js | 1 |  |
| jp |  | 1 |

.code

mov ax,07B1Ah

sub ax,0CEEBh

jc l1

L1: jz L2

L2: jo L3

L3: js L4

L4: jp L5

L5:

| Flags | Sign | 1 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 0 |  | 0111 1011 0001 1010 ans = -53D1 - 1100 1110 1110 1011 0011 0001 0001 0101 2’s complement Add m 0111 1011 0001 1010 1010 1100 0010 1111 again 2’s complement - 0101 0011 1101 0001 |
|  | Carry | 1 |  |  |
|  | Overflow | 1 |  |  |
|  | Parity | 0 |  |  |

Update flags after arithmetic instruction? Also state which of the following jumps will taken or not taken

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| Jnc |  | 1 |
| Jnz | 1 |  |
| Jo |  | 1 |
| Js | 1 |  |
| jnp |  | 1 |

.code

mov cx,0FBCDh

add cx,0E4AAh

jnc l1

L1: jnz L2

L2: jo L3

L3: js L4

L4: jnp L51

| Flags | Sign | 1 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 0 |  | 1 1 1 1 1 1 1 1 1 1 1111 1011 1100 1101 + 1110 0100 1010 1010 1110 0000 0111 0111 E 0 7 7 |
|  | Carry | 1 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 1 |  |  |
|  | Auxiliary | 1 |  |  |

- Update flags after arithmetic instruction? Also state which of the following jumps will taken or not taken

.code

mov bx,0FABDh

add bx,0684Ah

jc l1

L1: jz L2

L2: jno L3

L3: jns L4

L4: jp L5

L5: mov ah,04ch

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| Jc | 1 |  |
| Jz |  | 1 |
| Jno | 1 |  |
| Jns | 1 |  |
| jp |  | 1 |

| Flags | Sign | 0 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 0 |  | 1111 1 1111 1111 1010 1011 1101 + 0110 1000 0100 1010 0110 0011 0000 1010 1 6 3 0 7 |
|  | Carry | 1 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 0 |  |  |
|  | Auxiliary | 1 |  |  |

RENAME JUMPS: JE= JZ , JNZ= JNE

Update flags after CMP instruction? Also state which of the following jumps will taken or not taken

.code

mov bx,0A5A5h

not bx

cmp bx,05A5Ah

jz l1

L1: je L2

L2: jnz L3

L3: jnseL4

L4:

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| Jz | 1 |  |
| Je | 1 |  |
| Jnz |  | 1 |
| Jne |  | 1 |

| Flags | Sign | 0 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 1 |  | Not bx: 0101 1010 0101 1010 1010 0101 1010 0101 - 0101 1010 0101 1010 0101 1010 0101 1010 0000 0000 0000 0000 |
|  | Carry | 0 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 1 |  |  |
|  | Auxiliary | 0 |  |  |

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| CX | 4 | 3 | 2 | 1 | 0 |  |
| AX | 2 | 3 | 4 | 5 | 6 |  |

Update value of ax and cx registers after every iteration. Update any changes to the done to flag

| Flags | Sign | 0 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 1 |  |  |
|  | Carry | 0 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 1 |  |  |
|  | Auxiliary | 0 |  |  |

.code

mov ecx,5

mov ax,1

L1:

inc ax

dec ecx

jcxz end_loop

jmp L1

end_loop:

  - For two unsigned operands Zero and Carry Flag indicate the following relationship

  - For two signed operands Zero and Carry Flag indicate the following relationship

| Flags | Sign | 1 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 0 |  |  |
|  | Carry | 1 |  |  |
|  | Overflow | 1 |  |  |
|  | Parity | 0 |  |  |
|  | Auxiliary | 0 |  |  |

| Flags | Sign | 1 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 0 |  |  |
|  | Carry | 0 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 1 |  |  |
|  | Auxiliary | 0 |  |  |

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| Jnl |  | 1 |
| Jnle |  | 1 |
| JL | 1 |  |

Fill flag after every CMP instruction

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| Ja |  | 1 |
| Jg | 1 |  |

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| JNG |  | 1 |
| JNGE |  | 1 |
| JGE | 1 |  |

| Flags | Sign | 0 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 0 |  |  |
|  | Carry | 1 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 0 |  |  |
|  | Auxiliary | 1 |  |  |

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| JG |  | 1 |
| JNL | 1 |  |
| JLE |  | 1 |

| Flags | Sign | 0 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 1 |  |  |
|  | Carry | 0 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 1 |  |  |
|  | Auxiliary | 0 |  |  |

|  | TAKEN | NOT TAKEN |
| --- | --- | --- |
| JL |  | 1 |
| JNG | 1 |  |
| JGE |  | 1 |

| Flags | Sign | 0 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 1 |  |  |
|  | Carry | 0 |  |  |
|  | Overflow | 0 |  |  |

| Example ID |  | 23I - 3812 |
| --- | --- | --- |
| First digit of Your ID | ID1 | 3 |
| Second digit of Your ID | ID2 | 8 |
| Third digit of Your ID | ID3 | 1 |
| Fourth digit of Your ID | ID4 | 2 |

INSTRUCTIONS: (Question 15-16)  Dry run the program for each iteration with following consideration

- Dry run program gives below update table for each iteration count total number of iterations took by the program

ASM Program

.data data dw 66h,id1,045h,id2,040h,id3,-025h,id4,-010h,011h swap db 0 .code main PROC start: mov swap,0 ;Reset swap flag to no swap mov ebx,0 ;initialize array index to zero loop1: mov ax,[ebx+data] ;load number to ax cmp ax,[ebx+data+2] ;compare with next number jbe noswap ;no swap if already inorder mov dx,[ebx+data+2] ;load second element in dx mov [ebx+data+2],ax ;store first number in second mov[ebx+data],dx ;store second number in first mov swap,1 ;flag that a swap has been done noswap: add bx,2 ;advance bx to next index cmp bx,18 ;are we at last index jne loop1 ;if not than compare next 2 cmp swap,1 ;check if swap has been done je start ;if yes than make another pass INVOKE ExitProcess,0 main ENDP END main

|  |  |  |  |  |  |  |  |  |  |  | COUNT | SWAP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66 | id1 | 45 | id2 | 40 | id3 | -25 | id4 | -10 | 11 |  |  |  |

| 66 | 00 | 45 | 04 | 40 | 06 | DB | 05 | F0 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 66 | 45 | 04 | 40 | 06 | DB | 05 | F0 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 45 | 66 | 04 | 40 | 06 | DB | 05 | F0 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 45 | 04 | 66 | 40 | 06 | DB | 05 | F0 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 45 | 04 | 40 | 66 | 06 | DB | 05 | F0 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 45 | 04 | 40 | 06 | 66 | DB | 05 | F0 | 11 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 45 | 04 | 40 | 06 | 66 | 05 | DB | F0 | 11 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 45 | 04 | 40 | 06 | 66 | 05 | DB | F0 | 11 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 00 | 45 | 04 | 40 | 06 | 66 | 05 | DB | 11 | F0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Dry run program gives below update table for each iteration count total number of iterations took by the program

ASM Program

.data data dw 66h,id1,045h,id2,040h,id3,-025h,id4,-010h,011h swap db 0 .code main PROC start: mov swap,0 ;Reset swap flag to no swap mov ebx,0 ;initialize array index to zero loop1: mov ax,[ebx+data] ;load number to ax cmp ax,[ebx+data+2] ;compare with next number jge noswap ;no swap if already inorder mov dx,[ebx+data+2] ;load second element in dx mov [ebx+data+2],ax ;store first number in second mov[ebx+data],dx ;store second number in first mov swap,1 ;flag that a swap has been done noswap: add bx,2 ;advance bx to next index cmp bx,18 ;are we at last index jne loop1 ;if not than compare next 2 cmp swap,1 ;check if swap has been done je start ;if yes than make another pass INVOKE ExitProcess,0 main ENDP END main

|  |  |  |  |  |  |  |  |  |  |  | COUNT | SWAP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66 | id1 | 45 | id2 | 40 | id3 | -25 | id4 | -10 | 11 |  |  |  |

- Modify the program in QUESTION#15 for following data declaration.

data dw  66h,id1,045h,id2,040h,id3,-025h,id4,-010h,011h

swap db  0

- Fill and clear stack using following code

.data

string1 BYTE “Computer Organization & Assembly language”,0

len_str1 = ($ - string1 - 1)

| FILL STACK |  |  | STACK AT END OF PROGRAM |  |
| --- | --- | --- | --- | --- |
| 00C6 |  |  | 00C6 |  |
| 00C8 |  |  | 00C8 |  |
| 00CA | 00 0e |  | 00CA |  |
| 00CC | 00 00 |  | 00CC |  |
| 00CE | 00 0g |  | 00CE |  |
| 00E0 | 0000 |  | 00E0 |  |
| 00E2 |  |  | 00E2 |  |
| 00E4 | …………… |  | 00E4 |  |
| 00E6 | ……. |  | 00E6 |  |
| 00E8 | 00 0u |  | 00E8 |  |
| 00EA | 00 00 |  | 00EA |  |
| 00EC | 00 0t |  | 00EC |  |
| 00FE | 00 00 |  | 00FE |  |
| 00F0 | 00 0p |  | 00F0 |  |
| 00F2 | 00 00 |  | 00F2 |  |
| 00F4 | 00 0m |  | 00F4 |  |
| 00F6 | 00 00 |  | 00F6 |  |
| 00F8 | 00 0o |  | 00F8 |  |
| 00FA | 0000 |  | 00FA |  |
| 00FC | 00 0C |  | 00FC |  |
| 00FE | 00 00 |  | 00FE | empty |

.code

main PROC

mov ecx, len_str1

mov esi, OFFSET string 1

L1:

mov eax, 0

mov al, [esi]

inc esi

PUSH eax

LOOP L1

mov ecx, len_str1

mov esi, OFFSET string 1

L2:

mov eax, 0

pop eax

mov [esi], byte PTR al

inc esi

LOOP L2

INVOKE ExitProcess,0

main ENDP

END main

.data

Wvall WORD 0FFFFH

saveFLag WORD 0

.code

mov ax, 0

mov ax, Wvall

add ax,1

PUSHF                     ;store flag register

POP saveFlag              ;Cannot move flag register

;value to other register/or to memory

;Push Flag & pop to local could be used

mov bx,saveFlag

PUSHF

mov cx, 0FFFFH

add cx, 1                 ;will update flag register

POPF                      ;Restore value saved on stack to flag register

Dry run the assembly code given below and update flag register after executing POPF

| Flags | Sign | 0 |  | Calculation |
| --- | --- | --- | --- | --- |
|  | Zero | 1 |  | 1 1 1 F F F F + 0 0 0 1 0 0 0 0 |
|  | Carry | 1 |  |  |
|  | Overflow | 0 |  |  |
|  | Parity | 1 |  |  |

- Update stack after running assembly code given below

.data

.code

mov ax,1

mov cx,2

mov dx,3

mov bx,4

mov bp,5

mov si,6

mov di,7

PUSHA

mov ax,15

mov cx,14

mov dx,13

mov bx,12

mov bp,11

mov si,10

mov di,9

POPA

| FILL STACK |  |  | STACK AT END OF PROGRAM |  |
| --- | --- | --- | --- | --- |
| 00F0 |  |  | 00F0 |  |
| 00F2 | 00 07 |  | 00F2 |  |
| 00F4 | 00 06 |  | 00F4 |  |
| 00F6 | 00 05 |  | 00F6 |  |
| 00F8 | 00 04 |  | 00F8 |  |
| 00FA | 00 03 |  | 00FA |  |
| 00FC | 00 02 |  | 00FC |  |
| 00FE | 00 01 |  | 00FE | empty |

.data

.code

Sum PROC

add ax,bx

add ax,cx

RET

Sum ENDP

Main PROC
	mov ax,1

mov bx,2

mov cx,3

call sum

Main ENDP

| FILL STACK |  |  | STACK AT END OF PROGRAM |  |
| --- | --- | --- | --- | --- |
| 00F0 |  |  | 00F0 |  |
| 00F2 |  |  | 00F2 |  |
| 00F4 |  |  | 00F4 |  |
| 00F6 |  |  | 00F6 |  |
| 00F8 |  |  | 00F8 |  |
| 00FA |  |  | 00FA |  |
| 00FC |  |  | 00FC |  |
| 00FE | Next instruction (Main ENDP) |  | 00FE | empty |

22. Update stack after running assembly code given below

.data

val1 DW 5

val2 DW 6

.code

Sum PROC

PUSH ebp

mov ebp,esp

mov ax,0

add ax,[ebp+4]

add ax,[ebp+6]

POP ebp

RET 4

Sum ENDP

Main PROC
	Push val1

Push val2

call sum

Main ENDP

| FILL STACK |  |  | STACK AT END OF PROGRAM |  |
| --- | --- | --- | --- | --- |
| 00F0 |  |  | 00F0 |  |
| 00F2 |  |  | 00F2 |  |
| 00F4 |  |  | 00F4 |  |
| 00F6 |  |  | 00F6 |  |
| 00F8 | Ebp |  | 00F8 |  |
| 00FA | Next instruction |  | 00FA |  |
| 00FC | 06 00 |  | 00FC |  |
| 00FE | 05 00 |  | 00FE | empty |

| FILL STACK |  |  | STACK AT END OF PROGRAM |  |
| --- | --- | --- | --- | --- |
| 00C6 |  |  | 00C6 |  |
| 00C8 |  |  | 00C8 |  |
| 00CA |  |  | 00CA |  |
| 00CC |  |  | 00CC |  |
| 00CE |  |  | 00CE |  |
| 00E0 |  |  | 00E0 |  |
| 00E2 | Bp |  | 00E2 |  |
| 00E4 | Line 23 |  | 00E4 |  |
| 00E6 | Ax = 0 |  | 00E6 |  |
| 00E8 | Bp |  | 00E8 |  |
| 00EA | Line 23 |  | 00EA |  |
| 00EC | Ax = 1 |  | 00EC |  |
| 00FE | Bp |  | 00FE |  |
| 00F0 | Line 23 |  | 00F0 |  |
| 00F2 | Ax = 2 |  | 00F2 |  |
| 00F4 | Bp 00 |  | 00F4 |  |
| 00F6 | Main endp |  | 00F6 |  |
| 00F8 | A0 = 3 |  | 00F8 |  |
| 00FA | Ax |  | 00FA | Ax |
| 00FC | Dx |  | 00FC | Dx |
| 00FE | Cx |  | 00FE | Cx |

24. Convert the program given in Question#15 to a procedure where it receives array offset and array size

through STACK. It also creates a local variable for swap

.data

data dw  66h, 0x045, 40h, -025h, -010h, 11h  ; Sample array data

.code

main PROC

; Push parameters onto the stack

push OFFSET data        ; Push the address of the data array

push 6                  ; Push the number of elements in the array (adjust as needed)

; Call the BubbleSort procedure

call BubbleSort

; Exit program

INVOKE ExitProcess, 0

main ENDP

BubbleSort PROC

push ebp                ; Preserve base pointer

mov ebp, esp            ; Set up stack frame

sub esp, 4              ; Allocate space for local variable 'swap'

mov [ebp-4], 0          ; Initialize swap flag to 0 (no swap)

; Load parameters from stack

mov esi, [ebp+8]        ; Load array offset into ESI

mov ecx, [ebp+12]       ; Load array size into ECX

start:

mov ebx, 0              ; Initialize array index to 0

loop1:

mov ax, [esi + ebx]     ; Load current element into AX

cmp ax, [esi + ebx + 2] ; Compare with next element

jbe noswap              ; Jump if in order

; Swap elements if out of order

mov dx, [esi + ebx + 2] ; Load second element into DX

mov [esi + ebx + 2], ax ; Store first element in the second position

mov [esi + ebx], dx     ; Store second element in the first position

mov [ebp-4], 1          ; Set swap flag to 1

noswap:

add ebx, 2              ; Advance to next element

cmp ebx, ecx            ; Check if at last index

jne loop1               ; Repeat if not

; Check if a swap was made; if yes, repeat sorting pass

cmp [ebp-4], 1

je start

; Clean up stack frame

add esp, 4              ; Release local variable space

pop ebp                 ; Restore base pointer

ret 8                   ; Clean up parameters (2*4 bytes) and return

BubbleSort ENDP

END main